Feature Description Document (FDD), RAVE

Zii Document Number:

28-5018-10

Revision A

Revision History

| Revision | By | Date | Reason for Revision |
| --- | --- | --- | --- |
| A | L. DeNezzo | 9 Aug 2017 | Initial Release |

Document Source Location (at time of authoring):

Arena document

${  "#document_info":{

"object_ids":[

{

"prefix":"SRD-RCN-",

"next_id":8086

}

],

"schemas":[

{

"title":"#shortform",

"type":"object",

"properties":{

"id":{

"type":"string"

},

"method":{

"type":"string",

"maxLength":1,

"pattern":"^(I|A|D|T|X)$"

},

"old_id":{

"type":"string"

},

"satisfies":{

"type":"array",

"items":{

"type":"string"

}

},

"type":{

"type":"string",

"maxLength":3

}

},

"required":[

"id",

"method"

],

"additionalProperties":false

},

{

"title":"#section",

"type":"object",

"properties":{

"features":{

"type":"array",

"items":{

"type":"string"

}

}

},

"additionalProperties":false

},

{

"title":"#requirement",

"type":"object",

"properties":{

"features":{

"type":"array",

"items":{

"type":"string"

}

}

},

"additionalProperties":false

}

]

}

}$

Table of Contents

1	Overview	8

2	RAVE Features - Passenger	9

2.1	Display Control	9

2.2	Volume Control	9

2.3	PAX UI Language Selection	9

2.4	Content Display Filtering and Selection	10

2.5	Home Screen	10

2.6	Menu Application	10

2.7	Passenger Guide	10

2.8	Audio Application	10

2.9	Video Application	10

2.10	Audio Favorites	11

2.11	Video Favorites	11

2.12	Passenger Help	11

2.13	eReader Application	11

2.14	Catalog Application	11

2.15	News Application	11

2.16	Games Application	12

2.17	Survey Application	12

2.18	Mixed Media Application	12

2.19	Do Not Disturb	12

2.20	Passenger Announcements	12

2.21	Themes	12

2.22	Holy Quran Application	13

2.23	Premium Content Widget	13

2.24	RDU Illumination	13

2.25	Personal Electronic Device (PED) Pairing Application	13

2.26	Usage Data Capture	13

2.27	Basic Passenger Control Unit (PCU-B)	13

2.28	Tethered Passenger Control Unit (PCU-T)	14

3	RAVE Features - Passenger and Crew	16

3.1	Entertainment Control	16

3.2	Passenger Service System (PSS) Controls	17

3.3	Broadcast Audio	17

3.4	Broadcast Video	17

3.5	Welcome Monitor	17

3.6	Screen Saver	18

3.7	RAVE Moving Map Application	18

3.8	Betria Moving Map Application	18

3.9	External Camera Display	18

3.10	IPTV Display	18

3.11	Decompression Event Handling	18

3.12	Flight Information	19

3.13	Connecting Gate Information	19

3.14	Cabin Messaging	19

3.15	Seat Display Unit (SDU) Enable/Disable	19

3.16	External Video Input	19

3.17	Display Control Button	20

3.18	Pay Per Access via Code Entry	20

4	RAVE Features - Crew	20

4.1	Flight Data Entry	20

4.2	Crew Application Panel	20

4.3	Crew Settings Panel	20

4.4	Monitor Cabin Broadcasts	20

4.5	Flight Scripting	20

4.6	Cabin Lighting Control	21

4.7	Overhead Display Control	21

4.8	Parental Lock	21

4.9	Air / Ground Messaging	21

4.10	PRAM Playlist	21

4.11	RAVE Wireless Web Access	21

5	RAVE Features - Crew and Maintenance	21

5.1	Crew Panel Password Entry & Validation	21

5.2	Crew Validation via Card Entry	22

5.3	Multiple Crew Panels	22

5.4	System Status	22

5.5	Cell Modem Status	22

5.6	Crew Action Recording	22

5.7	CP-Disconnected / Reconnect	23

5.8	SCU-Disconnected / Reconnect	23

6	RAVE Features - Maintenance	23

6.1	Maintenance Application Panel	23

6.2	Aircraft ID Entry	23

6.3	Content Block	23

6.4	System Configuration	23

6.5	Software Load	24

6.6	Configuration Load	25

6.7	Content Load and Status	25

6.8	Cloud Load	25

6.9	Media Detection	26

6.10	System Conformity	26

6.11	Aircraft Data Transfer	26

6.12	Replaced RDU Tracking	26

6.13	Application Health Monitor	26

6.14	Built-In Test (BIT)	27

6.15	Network Status	27

6.16	Fault History	27

6.17	Maintenance Test Flight	27

6.18	Intrusive Seat BIT	27

6.19	Intrusive System BIT	27

6.20	Print Reports	27

6.21	Maintenance Port	27

6.22	Maintenance Utility Mode	28

6.23	System Time Management	28

7	RAVE Features - System	28

7.1	System Redundancy	28

7.2	SCU Configurable Interfaces	30

7.3	System Data Capture	30

7.4	LRU Startup	30

7.5	Thermal Management	30

7.6	SDU Stowed / Deployed	31

7.7	Acceptance Test Procedure	31

7.8	LRU Watchdog	31

7.9	Power Management	31

7.10	Power Management System Level Requirements	31

7.11	LRU Bus Integrity	31

7.12	Password Security	31

8	Line Replaceable Units (LRUs)	31

8.1	Basic Remote Jack Unit (RJU-B)	31

8.2	Electrically Accessible USB Device (EA-USB Device)	31

8.3	Remote Peripheral Unit (RPU)	32

9	Zii Internal	32

9.1	LRU Production Initialization	32

9.2	Content Licensing	32

9.3	Bench Maintenance	32

9.4	Content Creation	32

9.5	Product Verification Test	32

9.6	Environment Stress Screening	32

9.7	Miscellaneous	32

10	Third-Party LRUs	33

10.1	Niceview Flight Track Server	33

10.2	In-Seat Power Supply	33

10.3	Quick Access Button (QAB)	33

10.4	Airshow 420 Cabin Video Information System (CVIS)	33

10.5	Media Center Moving Map	33

11	Custom Features	33

11.1	Video In as VA Source	33

11.2	CIDS Proxy for Third-Party IFE	34

11.3	NIU as a Proxy for Pre-Standard Interface Boeing PSS	34

11.4	LIU as a Proxy for Pre-Standard Interface Boeing PSS	34

11.5	Embedded Niceview Moving Map Application	34

Appendix A	Acronyms	35

List of Tables

Table A-1 : Acronyms	36

List of Figures

Figure 3-1 : Redundant SCU System Diagram	8

Figure 3-2 : Single SCU System Diagram	9

Figure 4-1 : PCU-B Buttons	14

Figure 4-2 : RAVE Standard PCU-T	15

Figure 4-3 : RAVE Standard PCU-T2	15

Figure 4-4 : Example Custom PCU-T	16

Figure 9-1 : SCU1 Redundancy Diagram	28

Figure 9-2 : SCU2 Redundancy Diagram	29

Figure 9-3 : SCU3 Redundancy Diagram	30

Figure 12-1 : Quick Access Button	33

# Overview

The purpose of this document is to define the RAVE product, other than actual user interface capabilities, that require software implementation within the various RAVE LRUs.  The system capabilities are exposed through the "System Features" that make up the RAVE system.  This document groups these features by operational category or system significance.

The RAVE simple, seat-centric architecture is shown below.  Each seat is independent of all other seats so a failure in one seat does not impact any other seat. And since the seat does not depend on large complex networks and media servers to stream content, a failure of the SCU or the network does not cause any of the seats to fail to provide at least audio and video on demand.  The SCU provides basic functions that provide additional features for the passenger and crew. The SCU performs bulk content loads in the background, and can distribute video in real-time such as safety videos and boarding music via the gigabit Ethernet cabin network.  The SCU also provides the interface to the Aircraft Passenger Service System (PSS).  The crew panel SDU is identical and interchangeable with similar sized passenger SDUs. This provides a level of redundancy without additional part numbers or complexity.

The RAVE Standard GUI was developed using today's most popular personal devices as a guide.  The goal is to design a familiar experience for passengers, making them feel more comfortable with the GUI, thus enhancing their overall in-flight entertainment experience.

Figure 3-1 : Redundant SCU System Diagram

Figure 3-2 : Single SCU System Diagram

# RAVE Features - Passenger

## Display Control

The Display Control allows the user to control SDU brightness level and the On/Off functionality of the monitor.  Access to Display Controls will be available when Entertainment is On or Off.  The Display controls text will be displayed in the selected GUI Language or the default GUI Language and will disappear if the button is pressed again or after a specified time-out.

## Volume Control

Volume control allows the user to manage their audio entertainment.  Audio volume management will be available through the Passenger Control Unit (PCU) and the SDU.  Access to the volume settings will be available when Entertainment is On or Off.  The volume control settings text will be displayed in the selected GUI Language or the default GUI Language and access will disappear if the settings are pressed again or after a specified time-out.

## PAX UI Language Selection

The passenger is given a choice of languages to use when interacting with the SDU display.  The Language Selection Screen allows the passenger to select a language to populate the text items on the GUI screens.  The Language Selection Screen will come up as part of the startup sequence when Entertainment is turned On or when the Language icon is selected from the list of Applications or Settings.  If there is only one language provided, this screen may not be displayed.

## Content Display Filtering and Selection

The content presented to a passenger at any given seat can be filtered based on several different criteria.  Some of these criteria are determined at the time of content generation and some are determined on board the aircraft.

The passenger user interface offers various methods for selecting content.  An Initial selection is made based on parameters defined by the airline (see Content Metadata).  Content is further categorized in order to enable the passenger to quickly select the media they are interested in.  For videos that have multiple playback languages or subtitle languages, the passenger can choose the language they would like to hear or see.

## Home Screen

${"#section":{"features":["Rave Remix GUI Installed"]}}$

RAVE supports a customizable Home Screen containing flight status information, advertisements and airline selected media content.  Videos can be configured to play within the template framework, or play full screen.  The passenger will have access to all content, passenger service controls, passenger settings and flight status information from the Home Screen.

## Menu Application

The Menu Application is accessed using the Application (or App) or Menu button and is provided in a scrollable display of application icons.

## Passenger Guide

For media or application content that is configured for more than one item, a passenger guide will allow the passenger to select items based on all selections or sort selections by genre or other configurable categories.  For video selections, if supplied by the customer, the guide details will allow the passenger to watch a short preview of the video program.

For media or application content that is configured for a second level of categorization, a screen will allow the passenger to select to see only the titles within this second level of categorization.  This same guide can also be used for displaying items that include or don't include "Premium Content".

## Audio Application

The Audio Application provides on-demand playback capabilities to the passenger.  The passenger will be able to perform standard playback controls such as play, stop, pause, resume and seek.  The Audio Application provides the user with a list of audio albums. This feature also allows the user to create a personalized playlist of audio tracks from multiple albums as well as entire albums.  Audio can play in the background while passengers access other applications.  A 'now playing' feature allows the user to view the currently playing audio track and access audio controls from any screen.  RAVE supports content created using the latest revision of the Audio and Video Encoding Specification (28-5021-00) to optimize the onboard experience.

## Video Application

The Video Application provides on-demand playback capabilities to the passenger.  The Video Application feature allows the passenger to watch a full screen video playback.  For some system configurations, the passenger has the ability to select audio language at the beginning of the video, and change it at any time during video playback.  The same is true for subtitle selection.  The passenger will have the ability to stop/pause/resume a video as well as select a specific position within the video to begin playback.  Volume controls will be available to the passenger.  RAVE supports content created using the latest revision of the Audio and Video Encoding Specification (28-5021-00) to optimize the onboard experience.

## Audio Favorites

${"#section":{"features":["Audio Playlists"]}}$

RAVE supports the ability to link pieces of media together to form a playlist.  RAVE supports a single Audio Playlist that enables a passenger to create their own personal set of audio tracks.  This allows quick access to those audio content items during the flight.  Once created, the playlist functions like any other audio title (album).

## Video Favorites

${"#section":{"features":["Video Favorites"]}}$

RAVE supports a single Video Favorites List that enables a passenger to create their own personal catalog of videos. Once created, the Video Favorites displays a collection of video content items the passenger has selected as their favorites.  This allows quick access to those video content items during the flight.

## Passenger Help

${"#section":{"features":["Help", "Rave Classic GUI Installed"]}}$

The Passenger Help feature consists of a sequence of graphic images that guide the passenger through displays on how to access the Passenger Settings, the Applications or on the use of any configured passenger control devices (e.g., PCU-B, PCU-T, and RJU).  The Passenger Help feature will be displayed when Entertainment is first turned On or when the Help icon is selected from the list of Applications.

## eReader Application

${"#section":{"features":["eReader Application Installed"]}}$

The eReader Application provides the passenger a means of reading magazine articles, daily news, publications like immigration/customs instructions, connecting gate information, duty free catalogs, meal menus, books, more complex examples such as in-flight magazines or newspapers, or other customer provided information.

The Publication Reader provides passengers a means of viewing PDF files.  The Image Reader provides the passenger a means of reading a set of JPG or PNG images.

## Catalog Application

${"#section":{"features":["Catalog Application Installed"]}}$

The RAVE Catalog Application provides an interface to the passenger for the browsing of retail items, meal menus and travel destinations. This application can have multiple instantiations on the seat back allowing the airline flexibility to display the content. Each item can have up to four images and / or videos providing a visual guide supported by an associated text description in the selected UI language. Each instantiation of the application is driven through its own meta-data allowing the airline to update and modify the content as needed without a software update.

## News Application

${"#section":{"features":["News Reader Installed"]}}$

The News Reader Application provides the passenger a means of viewing text based or video based news content, such as daily news articles.  The Reader will also allow the passenger to view images that may be contained within the article.

## Games Application

${"#section":{"features":["Games Installed"]}}$

The Games Application feature provides the passenger the capability to play various touchscreen and PCU-controllable games.  Once a game is started from the passenger guide selection, the game can be controlled using the controls provided by the game.  Controls may be supplied by screen touches or by PCU-T button presses.  The passenger settings can be accessed to allow the passenger to adjust the volume and brightness settings.

## Survey Application

${"#section":{"features":["Passenger Survey Installed"]}}$

The Survey Application allows the airline to capture passenger responses to an airline configurable set of questions and answer choices.

## Mixed Media Application

${"#section":{"features":["Mixed Media Installed (Remix)"]}}$

Mixed Media Application provides content for the passenger as different media types grouped as a collection.  A customized background image can be used to enhance the collection type.  Examples of this are Kids Zone and Airline specific information.

## Do Not Disturb

${"#section":{"features":["Do Not Disturb Installed (Remix)"]}}$

The passenger can choose from a pre-configured set of 'Do Not Disturb' options to display an indication to the crew not to be disturbed, wake for meals, etc.  Upon selection, a configurable image is displayed on the seat display.

## Passenger Announcements

During an announcement, any content being played will be paused.  For audio announcements, when the display is in use, an overlay will be displayed to indicate an announcement is currently in progress.  During a video announcement (VA), the backlight will be set to a minimum brightness level, and the selected video announcement will be played in the configured classes within the aircraft.  When the backlight is off and the display is not disabled, the backlight will be turned on.  When the current volume of the unit is below a configurable minimum level, the audio volume will be increased to the minimum level.

RAVE is notified when a Public Address (PA) announcement is in progress or not in each zone of the aircraft.  The SCU distributes this information to all SDUs.  The SDU may also receive this information directly from the aircraft via a discrete input.  Upon receipt of a PA Pause message, the SDUs in the affected zone(s) will pause the playback of currently playing media and inform the passenger that a public address announcement is in progress.  Paused media is resumed when PA Pause finishes.

For aircraft with multiple decks:

RAVE will support up to 6 PA and VA zones each (maximum of 3 zones per deck)

- RAVE will send a zonal PA announcement to all selected zones (upper and/or lower deck)

- The same VA can be sent to any combination of the zones at the same time.  Two unique VAs can be played, one on the Upper deck (to any combination of the zones) and one on the Lower Deck (to any combination of the zones) at the same time.

- The crew can select PRAM and BGM to be played in upper, lower or both decks

## Themes

${"#section":{"features":["Remove From All CRDs", "Theme Selection Installed (Remix)"]}}$

The Theme feature allows the SDU to be personalized for a given flight.  For each theme, the SDU will display in predefined colors and background images will change to coordinate with the selected theme.  Different themes can be selected for each route and each class.

## Holy Quran Application

${"#section":{"features":["Holy Quran Audio Application"]}}$

The Holy Quran is the central religious text of Islam.  This application dedicated to the Holy Quran lies primarily in airlines that service Muslims worldwide, particularly on their pilgrimage to Mecca and Medina, Saudi Arabia.  Since this is a sacred text, these audio files will be kept separate from all other audio content out of respect, thus the ability to add any of The Holy Quran tracks to a playlist or favorites is not accessible. Shuffling tracks is also disabled for a similar reason.

## Premium Content Widget

${"#section":{"features":["Pay Per Access"]}}$

The Premium Content Widget allows an airline to advertise and provide access to premium entertainment packages directly from the Home Screen.  The Premium Content Widget will be cognizant of the 'paid content' state and display airline configured content based on the pay per access status (no purchase made, content purchased, free or comped flight).  The Premium Content Widget will always launch the pay per access application allowing the passenger to view the purchased packages and associated content.

## RDU Illumination

${"#section":{"features":["RDU2 Mood Lighting"]}}$

RDU Illumination provides the ability to configure the color and brightness around the USB Port, Audio Jacks and lighting around the edges of the RDU.  Any color can be specified through the use of tri-colored LEDS.

## Personal Electronic Device (PED) Pairing Application

${"#section":{"features":["Second Screen"]}}$

When the RAVE REMIX product is installed in conjunction with a wireless cabin configuration an airline can configure the PED Pairing Application.  When configured, the PED Pairing application is enabled on the seatback, as well as within the RAVE Wireless Cabin Application (RWCA).  The PED Pairing application provides the passenger with a secure means to associate the passenger personal electronic device (PED) with the seatback.

## Usage Data Capture

Recording passenger usage data enables an airline to analyze how passengers interact with the system in an attempt to identify steps that can be taken to tailor future content sets or airline operations in a way that creates a more satisfied passenger.  This feature records passenger actions that demonstrate actual usage of the content and stimulate an SDU external interface.  Each passenger action that is recorded includes the date and time that the action was taken as well as additional data directly related to the action that was taken.

## Basic Passenger Control Unit (PCU-B)

${"#section":{"features":["PCU-B Installed", "PCU Installed (Any PCU)", "PSS Controls Installed"]}}$

The Basic Passenger Control Unit (PCU-B) provides the passenger access to Passenger Service (PSS) reading light and attendant call/cancel functionality.  The PCU-B is typically used in front row seats where access to these PSS functions from a standalone device is necessary when an arm-mounted SDU is stowed.

Figure 4-1 : PCU-B Buttons

## Tethered Passenger Control Unit (PCU-T)

${"#section":{"features":["PCU Installed (Any PCU)"]}}$

The Tethered Passenger Control Unit (PCU-T) can provide the passenger access to Passenger Service (PSS) reading light and attendant call/cancel functionality.  In addition, the PCU-T also provides navigation, selection, game, and volume controls. The PCU-T is typically used in first and business class seats as the primary means of interacting with the SDU.  The buttons on the PCU-T have a constant low cabin-light illumination to aid the passenger's usability when the cabin is dark.

### Standard PCU-T

${"#section":{"features":["Standard PCU-T Installed"]}}$

Figure 4-2 : RAVE Standard PCU-T

### Standard PCU-T2

${"#section":{"features":["Standard PCU-T2 Installed"]}}$

Figure 4-3 : RAVE Standard PCU-T2

### Custom PCU-T

${"#section":{"features":["Custom PCU-T Installed"]}}$

Figure 4-4 : Example Custom PCU-T

# RAVE Features - Passenger and Crew

## Entertainment Control

Passenger entertainment functionality is controlled by whether Entertainment is On or Off. When Entertainment is On passengers have full access to all SDU functionality as described in other features.

While Entertainment is Off, the system can be configured to display a set of airline custom pages.  The entire screen can be utilized for a custom passenger experience as designed by the airline.  The pages will be displayed as a single page or continually cycle multiple pages until Entertainment is turned On via the Crew Panel.  An airline can also choose to configure an Audio Playlist to be played in the background.  The crew can also command the seats to a configured directed display such as connecting gate information, moving map data or camera views.

While the Entertainment is On, the system can be configured to display a set of airline custom pages.  The pages will be displayed as a single page or continually cycle multiple pages until the passenger selects a different application.

The Entertainment control allows the crew to view current status of passenger entertainment and provides the ability for the crew to turn it on or off as well as select displays for the seats and overhead.

## Passenger Service System (PSS) Controls

${"#section":{"features":["PSS Controls Installed"]}}$

Passenger Service System Controls (reading light and call light) for RAVE are integrated as a standard feature into the graphical user interface at all seats.  Optionally, these controls may be available through a PCU located at the seat.  These control functions at the seat interface with aircraft systems which control the actual indicators within the aircraft.

RAVE supports the ability for crew members to individually monitor and control passenger attendant calls and reading lights.  The crew member will be able to perform the following functions:

- Toggle the Reading Light at a single seat

- Cancel the Attendant Call at a single seat

- Disable the Attendant Call function

- Enable the Attendant Call function

- Disable the reading light control

- Enable the reading light control

## Broadcast Audio

While most audio / video playback in RAVE occurs as passenger on-demand local playback, some types of media are still broadcast from the SCU in the traditional way (PRAMs, PAs, BGM).

RAVE supports the ability for crew members to control boarding music to the PA system as well as initiate manual PRAM messages from the crew panel.  An airline can configure to allow a repeat capability for audio announcements.  In addition, the system can be configured to play one or more PRAM messages upon the activation of the Decompression discrete. (Reference Decompression Event Handling for details).

When the crew activates a Public Address (PA) announcement, current audio or video announcements and boarding music will be paused in the cabin and on the monitor screen of the CP.  When the PA announcement is complete, the paused audio, video or boarding music will resume.  Any active broadcast media will stop if a new broadcast selection is made by the crew.

The PRAM Screen allows the crew member to access a list of available pre-recorded audio announcements, defined in content, to distribute throughout the cabin.

The Boarding Music Screen allows the crew member to access a list of available boarding music, defined in content, to distribute throughout the cabin.

## Broadcast Video

RAVE supports the ability for crew members to broadcast video announcements from the crew panel.  More than one video can be started through one user action when configured in metadata.  An airline can configure to allow a repeat capability for video announcements.

While most video playback in RAVE occurs as passenger on-demand local playback, some types of media are still broadcast from the SCU in the traditional way.  For certain video broadcasts (VAs) the passenger is "forced" to watch while in other video broadcasts (Moving Map) the passenger must explicitly select the broadcast to view it.

The Video Announcements Screen allows the crew member to access a list of video announcements, defined in content, to distribute throughout the cabin.

## Welcome Monitor

${"#section":{"features":["Welcome Monitor Installed"]}}$

Welcome monitors are broadcast monitors typically mounted at the entry door of the aircraft. These monitors allow the airline to provide information to the passenger related to the flight or aircraft (such as row numbers) when the passenger boards the aircraft. The welcome monitors are supported at up to two locations per aircraft and deck (a maximum of 4 locations on an A380) with control at the crew panel for the broadcast content which will be shown at each of the locations. The content for the welcome monitors is loaded as part of the resident content, similar to PA/VA/PRAM data. Any RAVE display unit can be configured as a welcome monitor using one of the four possible locations. Broadcast content displayed on the welcome monitors may be configured to be the same as other broadcast monitors on the aircraft during various phases of flight, the airline has flexibility on the configuration for what is displayed.

## Screen Saver

When the passenger display is idle for a configurable amount of time a screen saver will generally be activated and will turn the backlight off.  While in Flight Mode or when logged out, if the crew panel is idle for a configurable amount of time a screen saver will be activated.  A swipe of the display or press of the Display Control button will turn the backlight on.

## RAVE Moving Map Application

${"#section":{"features":["Rave Broadcast Moving Map Installed (AI)"]}}$

The RAVE Moving Map Application is a software feature that provides a similar passenger experience to the traditional broadcast moving map offerings.  Instead of using a dedicated head end map server LRU RAVE uses an embedded software solution.  The SCU interfaces with the aircraft to receive regularly updated navigation data.  This navigation information is, in turn, regularly supplied to the RAVE Moving Map Application for it to use in generating the current view for presentation.

The Moving Map Screen displays either a video that is provided from the SCU or an external Moving Map LRU or the web page provided by the RAVE Moving Map Application.

## Betria Moving Map Application

${"#section":{"features":["Betria Moving Map Installed"]}}$

The Betria moving map products will provide an interactive and broadcast 2D and 3D experience across all RAVE platforms.  The Betria map provides the passenger with a realistic and detailed rendering of their aircraft against full a 3D earth background of satellite and street map visual layers. The passenger can watch different views of their flight progression on auto-play or interact with the map in a free roaming mode to explore the world they are flying over. Media enabled points of interest create the ultimate ‘infotainment’ service allowing the passenger to see information about hotels, restaurants, local tours, and more. Passengers can navigate the easy to use interface to explore destination cities down to the street level with named roads, buildings, bridges, parks and bodies of water.

## External Camera Display

${"#section":{"features":["Camera Installed (Any-Digital or Analog)"]}}$

RAVE can be configured to interface with an external Digital Camera LRU to receive one or more digital video streams and broadcast the stream(s) to the seats.

The Camera Screen displays a video that is provided from the SCU or an external Camera LRU.

## IPTV Display

${"#section":{"features":["exTV Installed"]}}$

RAVE has the capability to interface with an external Panasonic eXTV LRU to receive a configurable number of video streams.  This feature allows the passenger to watch a full screen video from a live video stream.  Volume controls will be available to the passenger.

## Decompression Event Handling

${"#section":{"features":["Decompression PRAM or Shutdown Processing Active"]}}$

Decompression shutdown processing occurs when an aircraft discrete informs the SCU that a cabin decompression event has occurred.  RAVE performs decompression shutdown processing on those LRUs that require immediate high-voltage shutdown due to concerns about these electronics.  This processing, as is true for all RAVE processing, has been developed to meet DO-178B Level E processes.  In addition to this software processing, all RAVE LRUs that contain high-voltage electronics also contain barometric pressure sensors that will automatically power-down the high-voltage electronics independent of any software intervention.

During a decompression event, an airline can configure whether to turn all SDU backlights off, leave the backlight on and display a configurable graphic to the passenger, leave the backlight on and display a configurable graphic to the crew, or leave the displays active.

## Flight Information

The Flight Information feature provides some notifications from the system which will be displayed on the SDU user interface independent of the passenger's choice of entertainment.  Time to Destination, when available from the configured aircraft interface, could be displayed to the passenger or the crew independent of an active application.

## Connecting Gate Information

${"#section":{"features":["Connecting Gate Information Installed"]}}$

RAVE supports the ability to display connecting gate information either as a selectable application at a seat or as a directed display to overhead monitors.

## Cabin Messaging

${"#section":{"features":["Cabin Messaging Installed"]}}$

The Cabin Messaging feature provides the flight attendants the ability to send messages to an individual seat or a group of seats within the aircraft.  These cabin messages can be updated via content load.

Cabin messages will be displayed in the language selected by the passenger, if provided in the content.  Based on configuration data, the cabin message will display for a selected amount of time or until the passenger acknowledges receipt of the cabin message.  Priorities assigned to cabin messages will determine the level of impact to the passenger's entertainment.

## Seat Display Unit (SDU) Enable/Disable

An airline can configure to allow a Seat Display Unit (SDU) to be disabled and re-enabled by the crew during a flight.  Disabling a unit will turn off the backlight and disable any touch capability for the passenger.  All disabled seats will be re-enabled when the flight segment has ended.

RAVE supports the ability for crew members to turn the backlight off for an individual seat. The passenger will be able to re-enable the backlight at the seat.

An airline can configure the 'touch' capability of touch screens for a particular class on an aircraft.  When only the 'touch' capability is disabled, the PCU-T will allow the passenger full control of all capabilities.

RAVE supports the ability for crew members to individually enable or disable the "Touch" capability for an SDU.  This enable/disable action by the crew will be an override of the configured default setting per seat class.  All override settings will be reset back to default values when the flight segment has ended.

RAVE supports the ability for crew members to perform a reset of an SDU from the crew panel.  This reset from the crew panel will operate the same as if the reset was performed at the seat.

## External Video Input

${"#section":{"features":["External Video at SCU"]}}$

The system supports receiving external video inputs at the SCU and transmitting the received video to all SDUs.

## Display Control Button

The Display Control Button can be used as a signal to the user interface or as a means for resetting the SDU.  The Display Control Button provides the user the ability to power on/off the SDU.  For some system configurations, the Display Control Button can be used to access the passenger settings.

## Pay Per Access via Code Entry

${"#section":{"features":["Pay Per Access via Code Entry"]}}$

A Premium Package containing a set of "premium" content can be defined and offered to passengers.  For each seat class, the "premium" content can be designated as "free" or "for purchase".   Actual payment transactions are accomplished external to RAVE.

For the Premium Package, a crew member has the ability to allow a seat, a class of seats or the entire aircraft to have complimentary access to the package as well as disable/enable access to the package for a class of seats.

A Purchase Code Algorithm supporting Premium content access may be updated by an airline customer and exported to USB media.  Once the code has been exported, maintenance personnel can use the USB media to import the updated algorithm to their remaining aircraft.

The Purchase Code Update Screen allows maintenance personnel to update a Purchase Code Algorithm, export a Purchase Code Algorithm to a USB device and import a Purchase Code Algorithm on the aircraft.

# RAVE Features - Crew

## Flight Data Entry

The Flight Details Screen allows the crew member to view and change the current flight information.  If available, this information will be populated automatically from the aircraft interface, but can be overridden at any time by the crew member.

## Crew Application Panel

The Crew Application Panel is accessed using the Application (or App) button.  The Crew Application Panel is a scrollable list of application icons.

## Crew Settings Panel

The Crew Settings Panel provides the user a quick and easy way to access basic screen functions (Power and Brightness) at any time.

## Monitor Cabin Broadcasts

${"#section":{"features":["Monitor Cabin Broadcasts"]}}$

The Monitor Cabin Broadcasts Screen provides the "unforced" video broadcast from multiple video sources which can be monitored on the crew panel.  The video broadcasts are the same video streams available to the cabin and can be accessed from any page on the crew panel.  The user can then close the view, as desired.  There may be multiple video sources available to select from (Moving Map, Camera or Live TV).

## Flight Scripting

${"#section":{"features":["Flight Scripting Installed"]}}$

The goal of Flight Scripting is to develop automatic tasking of some key events (like enabling Entertainment, disabling Entertainment, switching to arrival mode on decent/landing, etc.) so that the system can operate autonomously during the flight.

Flight Scripting triggers are configured events that RAVE uses to transfer between flight phases.  Upon detection of a trigger, RAVE will perform pre-defined actions as customized by the customer.

Crew members are provided current state status and are able to enable or disable Flight Scripting via the Crew Panel screen.  In the event the Crew Panel is disconnected from the SCU, Flight Scripting operates autonomously, when enabled.

## Cabin Lighting Control

${"#section":{"features":["EMTEQ Cabin Lighting Installed"]}}$

Cabin Lighting Control allows the crew to control and monitor cabin lighting selections and settings from a web-based interface with an external EMTEQ LRU.

## Overhead Display Control

${"#section":{"features":["Broadcast-only SDU Installed"]}}$

The Overhead Display Control feature allows the cabin crew to control the broadcast monitors (bulkhead and overhead).  This feature provides the ability to status the overhead and bulkhead monitors, turn the monitors off and select day or night brightness settings.

## Parental Lock

${"#section":{"features":["Parental Lock"]}}$

Parental lock provides a crew member with the ability to restrict the media titles that are made available to a selected passenger seat.  The passenger indicates the maximum media title rating (e.g. G, PG, R, ...) that is to be allowed for the designated seat(s).  From the Crew Panel, the crew member selects the seat(s) to be restricted, and enters the requested maximum media title rating.  The designated SDUs are notified of the ratings restriction and the restricted media listing is reflected immediately on the SDU displays.  If the buyer chooses to only have a Parental Lock ON or OFF rather than the various ratings, the rating selection dialog will not be displayed.

## Air / Ground Messaging

${"#section":{"features":["ACARS Messaging Installed"]}}$

RAVE supports the ability for flight attendants to send/receive text messages with ground stations via ACARS.  Received messages can be displayed at the crew panel as well as printed to the RAVE Standard Printer, as needed.  Support exists for predefined message requests and free text messages.

ACARS is one method of Air/Ground Messaging

## PRAM Playlist

TBD

## RAVE Wireless Web Access

${"#section":{"features":["RAVE Wireless Installed", "Remove From All CRDs"]}}$

RAVE interfaces with a wireless system to offer in-cabin connectivity to passenger devices.  A crew member will control and monitor the system using a web based application.

# RAVE Features - Crew and Maintenance

## Crew Panel Password Entry & Validation

Password validation enables flight attendants to have secure access to the crew panel and maintenance personnel to have secure access to maintenance-unique functionality.

The Login Screen is displayed upon power up and communication with the SCU is established or when returning from screen saver mode.  From this screen the user can select either Flight Mode, Maintenance Mode, or access the RAVE Wireless System.

The Logout control on the Application Panel allows the crew to log out of the system and return to the Login Screen.

## Crew Validation via Card Entry

${"#section":{"features":["Crew Validation via Card Entry"]}}$

RAVE supports crew validation and access to the crew panel via crew card if the crew card ID has a valid numerical sequence.  The system stores the crew card ID when crew swipe a card using the magnetic stripe card reader connected to the crew panel. The system uses the logged crew ID's to perform validation checks when free services are provided during the flight. Any crew operation at the seat or to provide free services in Pay Per Access is only allowed if the crew card swiped at the seat or crew panel matches a valid crew id stored at the headend.  (NOTE:  Maintenance personnel do not log in with a card.  Maintenance personnel are required to enter a password.)

## Multiple Crew Panels

${"#section":{"features":["Multiple Crew Panels Installed"]}}$

RAVE can support multiple crew panels on a single aircraft (no limitation on quantity).  All cabin crew functionality is available from any crew panel on the aircraft.  Maintenance information can be displayed at any crew panel.  Crew panel content loading is limited to a specific crew panel.  All information and status display will be synchronized across all configured crew panels.  Each crew panel will require login using a common set of configured login credentials for the fleet.  Crew panels can be associated with a specific cabin or deck on the aircraft to focus the information and actions relevant to that cabin or deck but will not limit capability to that cabin or deck.

## System Status

The RAVE system has an extensive set of continuous built-in tests (CBIT) that execute while the system is in operation.  Test results are reported to the SCU and summarized for the purpose of aiding flight attendants in the operation of the system.

The System Status Screen provides a summary of the RAVE system including seats recommended to be replaced, those with parental lock active, seats with PSS Controls disabled and seats with missing titles.  The screen contains a scrollable LOPA of the aircraft displaying all SDUs.  An icon will also appear on the seat if any titles are missing, when parental lock is active, when the touch capability is disabled or when an SDU is in the disabled state.  The screen will also provide an indication of a seat reset when it has been commanded from the crew panel.

The Maintenance System Status Screen is displayed in Maintenance Mode upon successful login of the maintenance personnel.  This screen displays an overall status of the system along with detailed LOPA information.

## Cell Modem Status

RAVE will notify crew and maintenance personnel when the wireless cell modem is active and when data offload and/or content upload is in progress when the aircraft is at the gate and connected to the Terminal Wireless Network (TWN).  These notifications from the system will be displayed on the Crew Panel independent of a particular display.

## Crew Action Recording

${"#section":{"features":["Remove From All CRDs"]}}$

This feature records operational data commanded or received at an LRU for RAVE features that are logged to support post-flight maintenance, system trend analysis, passenger usage analysis and Fleet degradation.

## CP-Disconnected / Reconnect

When an error occurs that disconnects the SCU from the crew panel, the SCU will begin to function autonomously.  This autonomous functionality will provide simple Entertainment mode transitions.  Once the error is corrected the SCU will revert to the Entertainment On/Off state commanded by the crew panel.

If communication is lost between the Crew Panel and the SCU, the Crew Panel will indicate the loss of communication.  While communication is lost, the SCU will turn Entertainment On when the aircraft is not at the gate.

## SCU-Disconnected / Reconnect

When an error occurs that disconnects an SDU (including the crew panel) from the SCU, the SDU will begin to function autonomously.  The autonomous functionality at a seat will simply turn Entertainment On, if not already done so.  Once the error is corrected the SDU will revert to the Entertainment On/Off state commanded by the SCU.  At the SDU-CP the crew will be notified of the disconnected state and all functionality at the crew panel will be inhibited.

# RAVE Features - Maintenance

The Maintenance Mode will allow the maintenance personnel to conduct extensive trouble shooting, content and software loading, data offload, and system configuration.  During Maintenance Mode, the screensaver time-out functionality will be disabled until maintenance personnel log out.

## Maintenance Application Panel

The Maintenance Application Panel is accessed using the Application (or App) button.  The Maintenance Application Panel is a scrollable list of application icons.

## Aircraft ID Entry

Aircraft Id is used as a unique identifier for various actions throughout the system.   For example, the Aircraft Id is used to uniquely identify the files that contain data offloaded from the aircraft.

If possible, the system will retrieve the Aircraft ID from the aircraft interface.  If the Aircraft ID retrieved from the aircraft matches the Aircraft ID stored in the RAVE system no other action is taken.  If the Aircraft ID retrieved from the aircraft can be correlated with a specific Aircraft Installation Type, the new Aircraft ID is used.  In all other cases, maintenance personnel will be required to enter a valid Aircraft ID.

## Content Block

${"#section":{"features":["Content Block"]}}$

The Content Block Screen provides the ability to restrict the content available for an aircraft.  The maintenance member will be able to disable from the Crew Panel any content contained on the SD Cards inserted into the Primary SCU.  Maintenance personnel will be able to view a list of all content that has been disabled and also have the ability to re-enable content that has been disabled.

## System Configuration

### On-Aircraft Configuration

In order to minimize the complexity of the off-aircraft generated database, RAVE creates unique data during installation of the system.  Some of this data is generated automatically; some requires entry by an installer.

The Edit/Display Configuration Screen allows the maintenance personnel to enable the SDU Configuration Display or the SDU Configuration capability.

The SDU Configuration Utility Display Screen provides the maintenance personnel with the ability to configure all of the SDUs throughout the aircraft.

The SCU and LRU Configuration Utility Display Screens provide the maintenance personnel with the ability to configure the Ethernet interfaces for all of the SCUs and other LRUs on the aircraft.

If a docking station has not been configured (either after being replaced, or upon initial installation), the RDU Initial Configuration Screen will be displayed in order for the unit to be identified.

### Configuration Display

The Seat Display Unit (SDU) configuration display page will provide the current hardware / software part, and seat configuration information when requested by the operator.  The SDU Configuration Display will display the current hardware and software configuration information at each of the SDUs throughout the aircraft including the SDU-CP.  The System Control Unit (SCU) configuration display page will provide the current hardware and software configuration information when requested by the operator.  The Line Replaceable Unit (LRU) configuration display page will provide the current hardware and software configuration information when requested by the operator.

### Aircraft Configuration Database (ACD)

Once Physical Configuration Databases (PCD) are loaded onto the aircraft via SD card, maintenance personnel can activate a PCD for a given aircraft configuration.  Using an active PCD, maintenance personnel can validate the current aircraft configuration and when no mismatches between the PCD and the current aircraft configuration data exist, a baseline configuration is established by creating the Aircraft Configuration Database (ACD) associated with the active PCD.

The Configuration Database Selection Screen allows the maintenance personnel to view the current active database and update the database from a list of database configurations associated with the current Aircraft ID.

The Configuration Import/Export Screen allows maintenance personnel to import a configuration from a USB device or save a configuration to a USB device.

### Auto Configuration Feature

${"#section":{"features":["Auto Configuration of Aircraft"]}}$

Maintenance personnel can configure an aircraft automatically using an ACD created for a specific aircraft type.  The SCU and all seat displays need to be connected to the network and operational for a full cabin configuration.  An SCU connected to a partial set of seat displays or seats not connected to the SCU can be configured as partial cabin configurations.

Maintenance personnel can configure an aircraft automatically using an ACD created for a specific aircraft type. An indication of success and failure of Auto Configuration is made available on the Auto Configuration Screen to maintenance personnel for troubleshooting purposes.

### Initialize SDU Personality

Each RDU on the aircraft can take on different roles based on configuration data contained in its DEM.  In this way, spare RDUs can be used to replace an RDU of the same size on the aircraft if the RDU is a replaceable type.

## Software Load

Software Load consists of Software / Firmware / Configuration Loading and will allow the system LRUs to remain current during all phases of flight. When a version mismatch is detected by an LRU, it will request the mismatched or missing file from the SCU. The download will complete and the LRU will return to normal operation. The software downloads will complete in most cases with minimal passenger interruptions.

The Software / Configuration Load Screen will display a list of the software / configuration that is stored on each of the SCUs.  The screen will indicate any discrepancies between the software / configuration that is currently loaded for each SCU and the software stored on the removable SD card.  Each SDU will show progress of the Software Load or the Import Configuration Load, as applicable.

## Configuration Load

${"#section":{"features":["GSE Tool"]}}$

Configuration Loading allows for updating the configuration of the system independent of a software load.

The Software / Configuration Load Screen will display the configuration update that is stored on each of the SCUs.  Each SDU will show progress of the Import Configuration Load, as applicable.

## Content Load and Status

Content Metadata is static data associated with a piece of content.  This data is used by the system to enable the user to select a content item for playback and to enable the system software to locate the content item for execution.

The Content Load feature handles the orchestration of copying content files from the head-end content source to the seats in the background during normal aircraft operation.  A new "content set" is inserted into a SCU or Crew Panel via SD Card(s) or USB drive.  Multiple content sets are managed at the SCU and distributed to the seats in a manner that ensures the most up-to-date content is available as quickly as possible and that no seat is left without content.

Content Load Status provides crew and maintenance personnel the status of all loadable content sets.  This information provides sufficient detail to support LOPA-style displays at the Crew Panel.

The Content Load Screen allows maintenance personnel to view the content installed at each of the SCUs, view the current content load status, and force the load of a content set.  When USB content is being imported to the SD Cards on the SCU, the crew will be able to view the progress and any errors that might occur.  This screen will also display the contents of each of the SD cards installed in either the Primary or the Secondary SCU, as applicable.

Content can be imported from a USB device at any time, even if the maintenance person or cabin crew is not logged in.

## Cloud Load

${"#section":{"features":["Cloud Load Capability"]}}$

RAVE supports content that is stored locally at the seat and distributed within the RAVE network (such as on the SCU).  Content that is not stored locally is downloaded to the local display on demand and begins to play immediately.  The airline is able to configure their content set to define if content is to be stored locally at the seat or if it is to be stored in the cloud.  Metadata for all content is always stored locally at the seat.

Note: Should a seat be on an island (no network connection), cloud content is not available.

## Media Detection

${"#section":{"features":["Remove From All CRDs"]}}$

At power up or when a SD card or USB drive is inserted into the SCU or Crew Panel, the media will be automatically read and recognized by the system in order to facilitate automatic use of the media. A series of LEDs will illuminate based on the condition of the detected media.

RAVE will notify maintenance personnel when error conditions occur and provide controls to allow a user to handle these errors, if possible.

## System Conformity

The operator requested LRU hardware / software conformity report provides a graphical representation of the current system configuration. The conformity report will highlight the differences between the baseline configuration contained in the active Physical Configuration Database (PCD) and Aircraft Configuration Database (ACD) and the current configuration. The conformity report will include software part numbers as well as hardware part numbers, versions, and serial numbers.

The Conformity Screen allows maintenance personnel to verify that the equipment detected on the aircraft matches the information in the Configuration Database.  The Hardware Summary lists all of the hardware part numbers that were detected, a description, the number expected in the Configuration Database, the actual number found, and any discrepancies.  The Software Summary lists all of the software part numbers that were detected, a description, the number expected in the Configuration Database, the actual number found, and any discrepancies.  The Detailed LRU Summary will provide a LOPA layout identifying each of the seat SDUs, SCUs and those SDUs that have been designated as Broadcast Units, Hot Spares, etc.

## Aircraft Data Transfer

When an aircraft is at the gate, RAVE will automatically connect to the Terminal Wireless Network (TWN) and offload log information as well as upload new content.

Maintenance personnel are able to offload the data logged as part of the "Logging" feature using a removable USB stick.

The Data Offload Screen will list all of the currently saved flight log information.  The list will display the Date/Time, Flight Number, City Pair information and Size of the offload bundle for easy identification by the maintenance personnel.  Flights conducted during the Maintenance Test Mode will be identified as such.

## Replaced RDU Tracking

Replaced RDU Tracking feature provides maintenance personnel information about the RDU's that are replaced by cabin crew during a flight or by maintenance on ground. This feature uses the existing BIT infrastructure to monitor, report and display the status on the crew panel.  An airline customer can configure this feature to allow for Manual or Automatic Acceptance of RDU replacements.  Default is Automatic Acceptance.

## Application Health Monitor

${"#section":{"features":["Application Health Monitor"]}}$

Application Health Monitor provides maintenance personnel with the status of all required software applications within the system.  Status is provided on the current activity and past issues of each application.

## Built-In Test (BIT)

The RAVE system has an extensive set of continuous built-in tests (CBIT) that execute while the system is in operation.  Test results are reported to the SCU and ultimately made available to maintenance personnel for troubleshooting purposes.

Refer to BIT ICD (26-5002-00) for detailed BIT Information.

## Network Status

The Network Status feature displays a graphical representation of the network status of the RAVE System.  An LRU and network path will be color coded to indicate status (Good, Degraded, or Failed).  LRUs in the RAVE system that can be detected on the network and report status are represented.    Information provided includes whether an LRU is connected and communicating on the network as expected.  The network status will update on a continual basis and can also be 'refreshed' as needed.

## Fault History

${"#section":{"features":["Fault History Enabled"]}}$

The Fault History feature provides maintenance personnel BIT result information for a configurable number of flight segments.  This information can be used to analyze RAVE network or specific LRU issues over time.

## Maintenance Test Flight

This feature provides functionality to simulate a flight during maintenance mode.  Normal crew operation is possible along with maintenance operations.  This feature can be used by maintenance personnel to exercise the system using the operational software instead of BIT.

## Intrusive Seat BIT

Intrusive Seat BIT provides maintenance personnel with tests that can be executed at each SDU in order to validate various aspects of the SDU's functionality.

Intrusive BIT allows maintenance personnel to execute tests to validate various aspects of the SDU or SCU-unique capabilities of the system.  When the Intrusive Seat BIT is turned ON, all SDUs, with the exception of those designated as Crew Panel, will enter Intrusive BIT mode.

## Intrusive System BIT

Intrusive System BIT enables maintenance personnel to test several system-wide or SCU-unique capabilities within the system.

## Print Reports

${"#section":{"features":["Rave Standard Printer Installed"]}}$

RAVE interfaces with a RAVE Standard Printer to provide printing services over the Ethernet.  RAVE will notify the user when a printing error occurs and provide controls to allow a user to handle these errors, if possible.

## Maintenance Port

Maintenance Port enables the maintenance user to have secure access to system maintenance-unique functionality.  The Maintenance Port(s) are connected to the SDU-CP and/or SCU.

The Maintenance Port password is used to enable the Ethernet maintenance port connected to the crew panel or SCU.

## Maintenance Utility Mode

${"#section":{"features":["Remove From All CRDs"]}}$

The Maintenance Utility Mode provides maintenance user with access to functions that can be executed at each SDU in order to validate various aspects of the SDU's functionality.

## System Time Management

System time is maintained on the SCU within the system.  Crew and maintenance personnel will be allowed to update the system time, when necessary.

If the SCU cannot determine a valid system time upon power up, when maintenance mode is first entered, maintenance personnel will be required to enter a valid date and time before they can continue.

# RAVE Features - System

## System Redundancy

${"#section":{"features":["SCU Redundancy"]}}$

Redundant SCUs enable RAVE to provide a higher availability for its centralized functions such as PSS, PA Pause, Moving Map, VAs, and PRAMs.  Communication between the redundant SCUs is designed to ensure a deterministic switchover occurs.  Overall system configuration information is continuously updated at the Secondary SCU in order to limit the interruption in services when a switchover does occur.

Figure 9-1 : SCU1 Redundancy Diagram

Figure 9-2 : SCU2 Redundancy Diagram

Figure 9-3 : SCU3 Redundancy Diagram

## SCU Configurable Interfaces

The way in which the RAVE system communicates with aircraft subsystems differs from one aircraft type to another.  The physical ports on the SCU can be configured to interface with various aircraft interfaces.

## System Data Capture

Logging records and maintains RAVE flight information. The logs support post-flight maintenance, system trend analysis, and passenger usage analysis. Examples of logged information are: hardware health data, passenger usage data, UI commands, etc. A flight cycle is used to define the start and the termination of a logging session.  Logs are bundled and uniquely identified for offload analysis.

## LRU Startup

Upon the application of power, each LRU in the system with an active computing element will perform basic testing to ensure it can enter the system.  When these tests fail a visible indicator on the LRU will be illuminated, if possible.

## Thermal Management

${"#section":{"features":["Remove From All CRDs"]}}$

The Thermal Management feature protects the SDU from internal over or under temperature conditions. The SDU powers down its host processor when the reported temperature exceeds its operational range. The SDU remains powered off until the temperature returns to within the operational range.

## SDU Stowed / Deployed

${"#section":{"features":["Stow Sensor Installed"]}}$

This feature provides functionality that is unique to arm-mounted SDUs. These SDUs have unique operating requirements for when they are stowed due to the limited ventilation characteristics of the enclosure.  There are also various mechanisms which place the SDU in various orientations when it is stowed.  These variables must be taken into account when developing a capability to support arm-mounted SDUs.  The configuration option that an SDU will utilize will be configured into the DEM during "On-Aircraft SDU Configuration" and loaded during the "Initialize SDU Personality" features.

## Acceptance Test Procedure

${"#section":{"features":["Remove From All CRDs"]}}$

An Acceptance Test Procedure is a demonstration of an LRU's compliance to a subset of its hardware requirements.  The purpose is to show that the LRU is functioning properly and ready for its intended use.  This testing includes test of critical internal elements as well as all major external interfaces.

## LRU Watchdog

${"#section":{"features":["Remove From All CRDs"]}}$

Various timers exist in hardware and software in order to quickly detect faulty hardware or software and enable more robust operation of the system.

## Power Management

The Power Management feature manages circuit power for an LRU.  Information will be logged to monitor protection circuit component activity and lifespan.

## Power Management System Level Requirements

${"#section":{"features":["Remove From All CRDs"]}}$

Zii's design process must incorporate design steps that consider and minimize, even during a failure event, the likelihood of passenger observable aircraft events that result in: odor, smoke, arcing, sparking, threatening sounds, discoloration, excess heat, fire, discharge of fluids or particles, high voltages, melting, shocks, warping or cracking.

## LRU Bus Integrity

A communication bus on an LRU can be connected to one or more internal devices.  RAVE monitors the communication busses and the connected devices and attempts to recover them when they become non-responsive.

## Password Security

${"#section":{"features":["Remove From All CRDs"]}}$

RAVE has several types of passwords for personnel to have secure access for unique functionality. The "Global LRU Password" is used for root access to the LRUs.  The RAVE VPN passwords are passwords required for RAVE to access Zii VPN.

# Line Replaceable Units (LRUs)

## Basic Remote Jack Unit (RJU-B)

${"#section":{"features":["RJU-B Installed"]}}$

The Basic Remote Jack Unit (RJU-B) provides a USB hub and possible connection to a PCU.

## Electrically Accessible USB Device (EA-USB Device)

${"#section":{"features":["EA-USB Device Installed"]}}$

The EA-USB Device can be any one of a number of different USB devices that can be connected to an SDU.  These devices can be accessed electrically, for configuration information.  These devices include various RJUs as well as USB repeaters and other peripheral extension units.

## Remote Peripheral Unit (RPU)

${"#section":{"features":["RPU Installed"]}}$

The RPU is a custom implementation on the Lufthansa program that allows various peripheral devices to be connected to an SDU.

# Zii Internal

${"#section":{"features":["Remove From All CRDs"]}}$

## LRU Production Initialization

As part of the LRU manufacturing process each LRU is initialized with unique identifiers and for the SDU, its security hardware is enabled for operation.

## Content Licensing

Content licensing applies to all content that requires secure playback.  Requirements must be met in three separate system phases in order for content to be played to a passenger:

	RDU Manufacturing - Embeds the RDU security functions within each playback device.  The functions are the encryption/decryption methods; secure storage for keys, and support for "trusted" (signed) software applications.

	Content Integration - Secures the content so that it can be safely transported to each aircraft and playback device as well as be decrypted efficiently at the time of playback.

	Media Playback - Utilizes the preceding processes to provide a secure playback environment within the RDU.  Decrypted media is never stored in non-volatile memory nor is it ever completely in the clear even in RAM.

## Bench Maintenance

The Bench Maintenance feature defines characteristics of the system that aid the LRU repair process.  Some apply to the on-aircraft software while others apply to off-line software tools.

## Content Creation

Content creation is the process required to build a customer's digital entertainment.  The process begins with the post production labs and content service providers that deliver content and metadata to Zii.  The content integration team then enhances the metadata and integrates the media into a specific customer content set.  The integrated content is copied to SD cards for customer deployment.

## Product Verification Test

The Product Verification Test (PVT) is executed on the unit under test (UUT) when performing qualification testing of an LRU.  The purpose of a PVT is to exercise the LRU as it would be functioning in "normal operations".

## Environment Stress Screening

The Environmental Stress Screening (ESS) is executed on multiple LRUs of the same type as a factory test.  It may or may not be executed on all LRUs that are manufactured.  The purpose of ESS is to find "infant mortality" failures within LRUs.  The test exercises the LRUs in "normal operations".  During the test, power is cycled, temperature is cycled, and vibration is varied as necessary.

## Miscellaneous

Some functions in RAVE affect the system as a whole and are independent of other features.

RAVE interfaces with Ethernet connected LRUs to provide the customer with multiple configurations tailored for their system needs.  Communication to and from RAVE for each External LRU should be specific for the LRU.

# Third-Party LRUs

## Niceview Flight Track Server

${"#section":{"features":["Niceview Installed-LRU"]}}$

RAVE interfaces with an external NiceView Flight Track Server LRU to receive moving map video and control the operational modes of the LRU.

## In-Seat Power Supply

${"#section":{"features":["SPB Connected via RS-485", "Monitor SPB Status"]}}$

RAVE can be configured to interface with an external In-seat Power Supply LRU (SPB) for DC power.  These In-seat Power Supply LRUs provide an RS-485 designed to accept control/request messages from an IFE system and respond with BIT and status information.

## Quick Access Button (QAB)

${"#section":{"features":["QAB Installed"]}}$

The Quick Access Button provides the passenger access to reading light, media playback pause/play control, volume control, and attendant call/cancel button.  The QAB is typically used in first and business class seats as the primary means of interacting with the SDU.

Figure 12-1 : Quick Access Button

## Airshow 420 Cabin Video Information System (CVIS)

${"#section":{"features":["Airshow 420 Installed", "Airshow Control Installed (Any)"]}}$

RAVE interfaces with an external Airshow 420 LRU to receive moving map video and control the operational modes of the LRU.  The SCU receives moving map video via an external video interface and transmits the input video to all SDUs.  The control interface with the Airshow 420 is used to retrieve information about the LRU and configure the video output of the LRU.

## Media Center Moving Map

${"#section":{"features":["Media Center Installed"]}}$

RAVE interfaces with an external Media Center LRU to receive moving map video and retrieve information about the LRU.

# Custom Features

## Video In as VA Source

${"#section":{"features":["Video In As VA Source"]}}$

The system can be configured to receive a discrete input that controls whether to "force" the external analog video input as a broadcast video to all SDUs.  This allows integration with an external IFE system that is in control of the programming of overhead monitors.

## CIDS Proxy for Third-Party IFE

${"#section":{"features":["CIDS Proxy for 3rd Party IFE"]}}$

CIDS is only able to interface PSS functionality to one IFE system via ARINC 429.  RAVE can be configured to operate in parallel with a second IFE system.  In this configuration, RAVE is directly interfaced to CIDS and the second IFE system's CIDS interface is connected to RAVE.  In this way, RAVE is able to ensure both IFE systems' PSS functionality is fully operational.

## NIU as a Proxy for Pre-Standard Interface Boeing PSS

${"#section":{"features":["NIU Installed"]}}$

On early Boeing 767 aircraft, the PSS system did not utilize the Boeing Standard Interface for communication with the IFE system.  For these aircraft RAVE utilizes its Network Interface Unit (NIU) to communicate passenger service information to the appropriate reading lights and call lights. The NIU is also equipped to interface to the maintenance panel and switches on the aircraft that allow for testing of the reading and call lights.

## LIU as a Proxy for Pre-Standard Interface Boeing PSS

${"#section":{"features":["LIU Installed"]}}$

On early Boeing 747 aircraft, the PSS system did not utilize the Boeing Standard Interface for communication with the IFE system.  For these aircraft RAVE utilizes its Local Area Controller (LAC) Interface Unit (LIU) to communicate passenger service information to the appropriate reading lights and call lights. The LIU is also equipped to receive zonal PA information and supply virtual discrete inputs to the system.

## Embedded Niceview Moving Map Application

${"#section":{"features":["Niceview Installed-Embedded"]}}$

The Embedded Niceview Moving Map Application is a software feature that provides a similar passenger experience to the traditional broadcast moving map offerings.  Instead of using a dedicated head end map server LRU, RAVE uses an embedded software solution.  The SCU interfaces with the aircraft to receive regularly updated navigation data.  This navigation information is, in turn, regularly supplied to the Embedded Niceview Moving Map Application for it to use in generating the current view for presentation.

Embedded Niceview Software and Data will be provided on USB Media.  The information will be copied to and will execute from Internal Storage on the SCU.  Loading Embedded Niceview Software and Data will be independent of RAVE software and content load.

The Embedded Niceview Information Load Screen will display a list of the software or data that is stored on each of the SCU's internal storage.

# Blank

Appendix A	Acronyms

| Acronyms | Expansion |
| --- | --- |
| ACARS | Aircraft Communications Addressing and Reporting System |
| ACD | Aircraft Configuration Database |
| ARINC | Aeronautical Radio, Incorporated |
| BGM | Boarding Music |
| BIT | Built-In Test |
| CBIT | Continuous Built-In Test |
| CFU | Cabin Fiber Unit |
| CIDS | Cabin Intercommunication Data System |
| CP | Crew Panel |
| CVIS | Cabin Video Information System |
| DC | Direct Current |
| DEM | Dock Electronics Module |
| EA-USB | Electrically Accessible - USB |
| ESS | Environmental Stress Screening |
| GUI | Graphical User Interface |
| IFE | In-flight Entertainment |
| JPG, JPEG | Joint Photographic Experts Group |
| LAC | Local Area Controller |
| LED | Light Emitting Diode |
| LIU | Local Area Controller (LAC) Interface Unit |
| LOPA | Layout of Passenger Accommodations |
| LRU | Line Replaceable Unit |
| NIU | Network Interface Unit |
| PA | Public Address/Passenger Announcement |
| PAX | Passenger |
| PCD | Physical Configuration Database |
| PCU | Passenger Control Unit |
| PCU-B | Passenger Control Unit - Basic |
| PCU-T | Passenger Control Unit - Tethered |
| PDF | Portable Document Format |
| PED | Personal Electronic Device |
| PNG | Portable Network Graphics |
| PRAM | Pre-recorded Audio Message |
| PSS | Passenger Service System |
| PVT | Product Verification Test |
| QAB | Quick Access Button |
| RAM | Random-access Memory |
| RAVE | Reliable, Affordable, Very Easy |
| RDU | Removable Display Unit |
| RJU | Remote Jack Unit |
| RJU-B | Remote Jack Unit - Basic |
| RPU | Remote Peripheral Unit |
| RWCA | RAVE Wireless Cabin Application |
| SD | Secure Digital |
| SDU | Seat Display Unit |
| SCU | System Control Unit |
| SPB | Seat Power Box |
| SPU | Seat Power Unit |
| SPU3 | Seat Power Unit 3rd Generation |
| TBD | To Be Determined |
| TWN | Terminal Wireless Network |
| USB | Universal Serial Bus |
| UI | User Interface |
| VA | Video Announcement |
| VPN | Virtual Private Network |
| Zii | Zodiac Inflight Innovation |

Table A-1 : Acronyms